SAFERPLACES × CLIMERA / KRAEKEN

# Scenario matrix for the Boismont & Givors cross-validation

Forcing scenarios prepared for the joint flood-model comparison with Climera/Kraeken (contact: Priscille Béguin): extreme sea levels for the Boismont coastal test site and design rainfall for the Givors pluvial test site, across three return periods and two emission pathways at two horizons.

Status: NDA signed    Return periods: 5 · 10 · 100 yr    Pathways: historical · SSP2-4.5 · SSP5-8.5

Horizons: 2050 · 2080    Handover: Tangi from Nov. 2026

COASTAL Boismont Somme estuary, Baie de Somme. Macrotidal regime (≈9–10 m spring range): extreme water levels are tide-dominated, with Atlantic storm surges of ≈0.5–1 m becoming critical only in coincidence with spring high tide. The village sits in the bas-champs behind the estuary embankments — flooding is an overtopping / breach mechanism, so total water level is read against crest elevation. DEM IGN_RGE_ALTI_1M_235643 · EPSG:3857 — 180761.4, 6469750.8 : 190925.4, 6479754.8

PLUVIAL Givors Gier–Rhône confluence, south-west of Lyon. A catchment prone to convective flash floods (2008 and October 2024 events on the Gier). Design storms are estimated at two durations — 1 hour (convective peak) and 24 hours (event total) — from regional Gumbel parameters. DEM IGN_RGE_ALTI_1M_120421 · EPSG:3857 — 526276.6, 5711163.5 : 534834.6, 5719471.5

# Boismont — extreme sea level (ESL)

Static extreme water level at the reference port (Le Hourdel / Saint-Valery), datum IGN69. Sea-level-rise projections (IPCC AR6, global median) are added to the historical return level at each horizon. Values in metres (IGN69).

| Return period | Historical | SSP2-4.5 · 2050 | SSP2-4.5 · 2080 | SSP5-8.5 · 2050 | SSP5-8.5 · 2080 |
| --- | --- | --- | --- | --- | --- |
| 5 years | 5.05 | 5.24 | 5.39 | 5.26 | 5.49 |
| 10 years | 5.15 | 5.34 | 5.49 | 5.36 | 5.59 |
| 100 years | 5.45 | 5.64 | 5.79 | 5.66 | 5.89 |

Return-period collapse. The macrotidal extremes curve is flat (≈40 cm between the 5- and 100-year levels). Under SSP2-4.5 by 2080, the projected 5-year ESL (5.39 m) is almost indistinguishable from today's historical 100-year level (5.45 m) — a water level that currently recurs once a century would recur roughly every five years instead.

# Givors — design rainfall

Depth estimates from regional Gumbel parameters (1 h: μ=25 mm, CV=0.40; 24 h: μ=55 mm, CV=0.35), scaled for climate change at +7%/°C against south-east France warming of +1.3/+1.9 °C (SSP2-4.5, 2050/2080) and +1.6/+3.0 °C (SSP5-8.5, 2050/2080).

1-hour duration (mm)

| Return period | Historical | SSP2-4.5 · 2050 | SSP2-4.5 · 2080 | SSP5-8.5 · 2050 | SSP5-8.5 · 2080 |
| --- | --- | --- | --- | --- | --- |
| 5 years | 32.2 | 35.2 | 36.6 | 35.9 | 39.4 |
| 10 years | 38.0 | 41.5 | 43.3 | 42.4 | 46.6 |
| 100 years | 56.4 | 61.5 | 64.1 | 62.8 | 69.0 |

24-hour duration (mm)

| Return period | Historical | SSP2-4.5 · 2050 | SSP2-4.5 · 2080 | SSP5-8.5 · 2050 | SSP5-8.5 · 2080 |
| --- | --- | --- | --- | --- | --- |
| 5 years | 68.8 | 75.2 | 78.3 | 76.7 | 84.3 |
| 10 years | 80.1 | 87.5 | 91.1 | 89.3 | 98.1 |
| 100 years | 115.4 | 126.0 | 131.2 | 128.5 | 141.3 |

Sub-daily sensitivity. Short-duration convective rainfall can scale faster than the simple Clausius-Clapeyron rate (+7%/°C). At +14%/°C (“super-CC”), the 1-hour, 100-year, SSP5-8.5 2080 depth would reach ≈85 mm rather than 69.0 mm. The 24-hour totals are considered adequately represented by the simple scaling.

# Data sources & assumptions

- Boismont ESL: reference port statistics (Le Hourdel / Saint-Valery), IGN69 datum; sea-level-rise addenda from IPCC AR6 global medians for the eastern English Channel, negligible local subsidence.

- Givors rainfall: regional Gumbel fit for the Lyon area; climate-change scaling applied via Clausius-Clapeyron against south-east France regional warming trajectories.

- Elevation data: IGN RGE ALTI® 1 m DEM for both sites (EPSG:3857).

Figures to be cross-checked against SHOM/Cerema published return-level tables and Climera's own sea-level-rise dataset before final sign-off.

SaferPlaces

GECOsistema S.r.l. · Piazza Malatesta, 21 · 47923 Rimini (RN) Italia · platform@saferplaces.co